# Cleaning and disinfection of vehicles

This document is part of the standard operating procedures (SOPs) for porcine epidemic diarrhoea (PED) and is identified as SOP 7.

Thorough cleaning and disinfection of vehicles is essential to prevent the spread of disease between farms.

## Follow these steps to ensure effective cleaning and disinfection:

### 1. Remove debris from the vehicle exterior

Suggested equipment:

Hand‐held brush

Location:

Away from wash bay

Notes:

Remove any mud from wheels, mud ﬂaps and chassis

### 2. Clear the vehicle interior

Suggested equipment:

Rake/shovel

Broom

Pig board

Location:

Away from wash bay

Notes:

Remove footwell mats from cab

Remove all equipment from tote/belly boxes

Push bedding to the rear of each deck

Start at the top deck and work down

Scrape bedding into container or storage area – there must be no risk of dirty bedding draining towards wash bay

### 3. Apply detergent to the vehicle exterior

Suggested equipment:

Foaming lance – low/medium pressure

Brush/broom

Location:

Wash bay

Notes:

Ideally apply using lukewarm water

Work from front of vehicle to rear

Don’t forget storage areas, all decks of vehicle, wheels, wheel arches, mud ﬂaps and the underside of the vehicle

All stored equipment, e.g. pig boards, boots, etc., must also be included

Apply using methodical overlapping horizontal sweeps (vertical sweeps on vertical edges and corners)

Leave for a minimum of 10 minutes

Scrub stubborn deposits with brush/broom

### 4. Rinse off detergent

Suggested equipment:

Pressure washer – high pressure

Location:

Wash bay

Notes:

Rinse oﬀ all contaminated material and detergent

Use cold or lukewarm water

### 5. Leave to drain

Location:

Wash bay

Notes:

Allow enough time for excess water to drain away before moving to next step

Failure to do this will result in dilution of disinfectant below the recommended concentration

### 6. Apply disinfectant to the exterior

Suggested equipment:

Foaming lance – low/medium pressure

Location:

Wash bay

Notes:

Apply using lukewarm water

Apply at correct concentration, as per Defra guidance on GOV.UK – disinfectants.defra.gov.uk

Work from front to back and top to bottom

Do not touch disinfected areas with boots or hose

### 7. Apply detergent to the vehicle interior

Suggested equipment:

Foaming lance – low/medium pressure

Brush/broom

Location:

Wash bay

Notes:

Ideally apply using lukewarm water

Start on top deck and work down

On each deck, work from top to bottom (ceiling, walls, ﬂoors) and from front of vehicle to rear

Don’t forget corners, behind roll‐up doors, backs of doors and nipple drinkers

Apply using methodical overlapping horizontal sweeps (vertical sweeps on vertical edges and corners)

Leave for a minimum of 10 minutes

Scrub stubborn deposits with brush/broom

### 8. Rinse off detergent

Suggested equipment:

Pressure washer – high pressure

- Notes:

Rinse oﬀ all contaminated material and detergent

Work in same order as disinfectant was applied

Use cold or lukewarm water

Do not splash contaminated material onto previously rinsed areas or onto other vehicles

### 9. Leave to drain

Location:

Wash bay

Notes:

Allow enough time for excess water to drain away before moving to next step

Failure to do this will result in dilution of disinfectant below the recommended concentration

### 10. Apply disinfectant to the interior

Suggested equipment:

Foaming lance – low/medium pressure

Location:

Wash bay

Notes:

Apply using lukewarm water

Apply at correct concentration

Work from front to back and top to bottom

### 11. Clean the cab

Suggested equipment:

Vacuum cleaner

Brush

Location:

Wash bay

Notes:

Vacuum, wash with soap/detergent and disinfect all surfaces, including ﬂoor, pedals, steering wheel, gear stick, etc.

Clean, disinfect and dry the mats before replacing

Ensure all items that are brought into the cab are clean and disinfected

### 12. Rinse the concrete/wash bay

Suggested equipment:

Pressure washer – medium pressure

Location:

Wash bay

Notes:

Clean the concrete ﬂoor of the wash bay

Take care not to splash contaminated water onto vehicle

Follow Defra guidance on how to prevent pollution from cleaning and disinfection

### 13. Inspect the vehicle

Suggested equipment:

Clean white gloves (optional)

Location:

Wash bay

Notes:

Visually inspect vehicle for cleanliness

Wipe gloved ﬁnger over surfaces to check for cleanliness

### 14. Avoid recontamination

Location:

Away from wash bay

Notes:

Exit wash bay via a separate route to incoming soiled vehicles

Park away from soiled vehicles

### 15. Dry the vehicle

Location:

Away from wash bay

Notes:

Dry as completely and quickly as possible

If active drying with heaters is not possible, use sunlight and/or natural airflow

## Explore further resources on cleaning and disinfection

These sources provide additional information on cleaning and disinfection of vehicles:

Defra guidance on how to prevent pollution from cleaning and disinfection on GOV.UK – gov.uk/guidance/animal-disease-outbreak-prevent-pollution-from-cleaning-and-disinfection

All SOPs available on our website – ahdb.org.uk/PEDv